## 2.1. INSTRUCCIONES ASM DEL PIC16F84

## El microcontrolador PIC 16F84 tiene un total de 37 instrucciones de una sola palabra, y además otras 28 instrucciones especiales que se corresponden con combinaciones de 2 ó 3 instrucciones simples.

## Las instrucciones tienen letras relacionadas (parámetros) y que tienen una interpretación diferenciada.

## La letra “W” es el registro más importante que tiene el PIC y se le denomina ACUMULADOR, ya que las operaciones pasan a través de él.

## La letra “k” es un literal (valor numérico) que puede contener cualquier valor asignado por el programador entre 0 y 255 (es el máximo que se puede representar con un byte).

## La letra “f” es cualquier nombre dado a un registro.

## La letra “d” indica en que lugar se almacenará el resultado de la instrucción (en el acumulador “W” o en el registro “f”).

## Si d = 0,  el resultado se almacenará en “W”.

## Si d = 1, el resultado se almacenará en “f”.

### Instrucciones orientadas a los bytes

| ###### Mnemónico | ###### Parámetros | ###### Descripción | ###### Ciclos | ###### Banderas |
| --- | --- | --- | --- | --- |
| ADDWF | f, d | Add W and f | 1 | C, DC, Z |
| ANDWF | f, d | AND W with f | 1 | Z |
| CLRF | f | Clear f | 1 | Z |
| CLRW | - | Clear W | 1 | Z |
| COMF | f, d | Complement f | 1 | Z |
| DECF | f, d | Decrement f | 1 | Z |
| DECFSZ | f, d | Decrement f, Skip if 0 | 1(2) | None |
| INCF | f, d | Increment f | 1 | Z |
| INCFSZ | f, d | Increment f, Skip if 0 | 1(2) | None |
| IORWF | f, d | Inclusive OR W with f | 1 | Z |
| MOVF | f, d | Move f | 1 | Z |
| MOVWF | f | Move W to f | 1 | None |
| NOP | - | No Operation | 1 | None |
| RLF | f, d | Rotate left f through carry | 1 | C |
| RRF | f, d | Rotate right f through carry | 1 | C |
| SUBWF | f, d | Subtract W from f | 1 | C, DC, Z |
| SWAPF | f, d | Swap nibbles in f | 1 | None |
| XORWF | f, d | Exclusive OR W with f | 1 | Z |

### Instrucciones orientadas a los bits

| ###### Mnemónico | ###### Parámetros | ###### Descripción | ###### Ciclos | ###### Banderas |
| --- | --- | --- | --- | --- |
| BCF | f, b | Bit Clear f | 1 | None |
| BSF | f, b | Bit Set f | 1 | None |
| BTFSC | f, b | Bit Test f, Skip if Clear | 1 (2) | None |
| BTFSS | f, b | Bit Test f, Skip if Set | 1 (2) | None |

### Operaciones con literales y de control

| ###### Mnemónico | ###### Parámetros | ###### Descripción | ###### NroCic. | ###### Banderas |
| --- | --- | --- | --- | --- |
| ADDLW | k | Add literal and W | 1 | C, DC, Z |
| ANDLW | k | AND literal with W | 1 | Z |
| CALL | k | Call subroutine | 2 |  |
| CLRWDT | - | Clear Watchdog Timer | 1 | TO,PD |
| GOTO | k | Go to address | 2 | None |
| IORLW | k | Inclusive OR literal with W | 1 | Z |
| MOVLW | k | Move literal to W | 1 | None |
| RETFIE | - | Return from interrupt | 2 | None |
| RETLW | k | Return with literal in W | 2 | None |
| RETURN | - | Return from Subroutine | 2 | None |
| SLEEP | - | Go into standby mode | 1 | TO,PD |
| SUBLW | k | Subtract W from literal | 1 | C, DC, Z |
| XORLW | k | Exclusive OR literal with W | 1 | Z |

Las instrucciones TRIS y OPTION  recomienda Microchip no utilizarlas, para

mantener la compatibilidad con el PIC16CXX. Aunque eso no significa que

tengamos que hacer caso y no usarlas.

### Instrucciones Especiales

| ###### Mnemónico | ###### Parámetros | ###### Descripción | ###### Operación Equivalente | ###### Banderas |
| --- | --- | --- | --- | --- |
| ADDCF | f, d | Add Carry to File | BTFSC 3,0 INCF f,d | Z |
| ADDDCF | f, d | Add Digit Carry to File | BTFSC 3,1 INCF f,d | Z |
| B | K | Branch | GOTO k | - |
| BC | K | Branch on Carry | BTFSC 3,0 GOTO k | - |
| BDC | K | Branch on Digit Carry | BTFSC 3,1 GOTO k | - |
| BNC | K | Branch on No Carry | BTFSS 3,0 GOTO k | - |
| BNDC | K | Branch on No Digit Carry | BTFSS 3,1 GOTO k | - |
| BNZ | K | Branch on No Zero | BTFSS 3,2 GOTO k | - |
| BZ | K | Branch on Zero | BTFSC 3,2 GOTO k | - |
| CLRC |  | Clear Carry | BCF 3,0 | - |
| CLRDC |  | Clear Digit Carry | BCF 3,1 | - |
| CLRZ |  | Clear Zero | BCF 3,2 | - |
| LCALL | K | Long CALL | BSF/BCF 0A,3 BSF/BCF 0A,4 CALL k | - - - |
| LGOTO | K | Long GOTO | BSF/BCF 0A,3 BSF/BCF 0A,4 GOTO k | - - - |
| MOVFW | F | Move File to W | MOVF f,0 | Z |
| NEGF | f, d | Negate File | COMF f,1 INCF f,d | Z |
| SETC |  | Set Carry | BSF 3,0 | - |
| SETDC |  | Set Digit Carry | BSF 3,1 | - |
| SETZ |  | Set Zero | BSF 3,2 | - |
| SKPC |  | Skip on Carry | BTFSS 3,0 | - |
| SKPDC |  | Skip on Digit Carry | BTFSS 3,1 | - |
| SKPNC |  | Skip on No Carry | BTFSC 3,0 | - |
| SKPNDC |  | Skip on No Digit Carry | BTFSC 3,1 | - |
| SKPNZ |  | Skip on Non Zero | BTFSC 3,2 | - |
| SKPZ |  | Skip on Zero | BTFSS 3,2 | - |
| SUBCF | f,d | Substract Carry from File | BTFSC 3,0 DECF f,d | Z |
| SUBDCF | f,d | Substract Digit Carry from File | BTFSC 3,1 DECF f,d | Z |
| TSTF | f | Test File | MOVF f,1 | Z |